How to force slot 2 to use PCIe 2.0 on a Mac Pro 3,1

1.  Unzip the PCITools zip file and the Kext Drop zip file

2.  The PCITools.zip will create the following folders. Opt/pciutils/sbin

3.  Make a copy the following from the sbin folder and copy to your users library i.e. /Users/jonfarley

setpci

Fast.sh

Lspci

Update-pciids

This is important because the script fastpci will need these to be there when it runs

4.  Ensure SIP is disabled by running terminal and typing csrutil status

5.  Run Kext Drop file that was unzipped

6.  Now drag and drop the DirectHW.kext from opt/pciutils/sbin on to Kext Drop and click install

Ensure it installs OK

7. Open a terminal windows

8. Type

Sudo nvram boot-args=“debug=0x144”

Sudo ./update-pciids

Sudo ./lspci

And make sure your NVMe disk starts at address 01:00 as shown below.  If this is not the case edit fastpci in text editor to replace 00:1 with the correct number

9.  Copy the script fastpci to your /users library, i.e. /Users/jonfarley

10.  Double click the script to run

You should see an output that says something like:

Initial PCIe 1.0 x16 (or x4 if you have a 4 lane card)

Final PCIe 2.0 x16 (or x4 if you have a 4 lane card)

If you don’t see the change to 2.0 it has not worked.

Now run a disk speed check and you should be running a fast  PCIe NVMe disk.

However, this will require the fastpci script to run at login.

11.  Go to system preferences and click your user and Login Items

12.  Add the script to your login items

This will run the script in Terminal at each login.

You can adjust Terminal preferences so it closes when a script finishes.

You will need to enter you admin password when the script runs.  You can bypass this by turning off password requirements for any sudo command but it involves editing a system file, sudoers, which you may or may not wish to do, depending how much you hate having to enter a password.  If you get this wrong you may not be able to use the sudo command until it is put right (requires Recovery Mode)

To edit the file run a Terminal session and

Type cd /etc

Sudo visudo

Use the arrow keys to navigate down to the line starting %admin

Change the line to read

%admin             ALL = (ALL) NOPASSWD: ALL

As show below, Press I to go into Insert mode, R to go into Replace mode.  Escape exists both modes

Do not change anything else.

Type :wq to save and overwrite the file

Run the fastpci script again and it should no longer prompt for password.